Final Specifications for

Lot 39 Jeremy Cay

9 Lost Village Trail

Edisto Island, SC

John Geoffroy

1781 Smokerise East

Conyers, GA 30012

770.929.1589 home		678.637.3001 cell	770.457.7686 fax

| 1000 | # General The builder shall comply with all rules and regulations regarding construction as listed in the Architectural Review Board Guidelines. These pertain to entrance and exit through the gate, traffic control, hours of construction and jobsite cleanliness. |
| --- | --- |
| 1010 | # Lot The builder shall erect the house upon the owner’s lot. |
| 1020 | Permits The builder shall obtain and pay for all required permits. The owner shall submit all required plans and documents to the Jeremy Cay Property Owner’s Association Architectural Review Board and the owner shall provide all bonds and fees required. |
| 1030 | Warranty Builder shall provide a full 1 year warranty for all elements and systems of the house. Manufacturers warranties shall be transferred to the owner for all products after the one year period. Upon issuance of the Certificate of Occupancy by the County, Builder shall provide to the Owner a list of all manufacturers warranties, the expiration date and the contact person and phone number for warranty service. |
| 1040 | Insurance Builder shall carry worker’s compensation, builder’s risk and general liability insurance. Builder shall furnish insurance certificates of such coverage at contract signing. |
| 1100 | Blueprints The Owner shall furnish the Builder 7 sets of final blueprints at contract signing. Any changes that were made since the blueprints were printed for biding will be clearly marked in clouds and will be subject to price review or adjustment by the Builder. Any changes to the design during construction shall be reviewed and approved by the Owner’s structural engineer. |
| 1120 | Survey The Owner shall be responsible for obtaining and paying for all surveys. |
| 1410 | Electric Utilities Builder shall apply and pay for all electric power used during construction. Service shall be transferred to the Owner’s name upon issuance of the Certificate of Occupancy. |
| 1430 | Water Source and Treatment Builder shall drill well and install pump. Water treatment shall be reverse osmosis purifying system 500 gpd with 1,200 gal tank minimum. There shall be a 5 micron prefilter. Pump shall have stainless piping and fittings and shall have a salt rejection of 95-99%. Post RO output shall have an ultraviolet disinfection system, 3 gpm min flow. RO brine discharge must not be into surface drainage. |
| 1450 | Sewer Tap Builder shall provide grinder at waste line exit and shall tap into existing sewer outfall line. |
| 1600 | Cleanup Builder shall provide trash container of sufficient size and emptying frequency to prevent any accumulation of trash on the grounds. Grounds shall be policed at end of business daily to insure a clean jobsite. |
| 2000 | Grading and tree removal Palm trees must be relocated and replanted on the property or on another site. Trees close to the house must be carefully trimmed prior to pile driving to insure their health and survival. All trees within the house footprint to be removed will be spray painted by the owner to be clearly marked. The owner or owner’s surveyor will mark the general location of the house (+- 24inches) so that a pad can be built of appropriate bearing material 2 feet surrounding the footprint. |
| 2030 | Grade Patios and Drive The drive and walks will be laid out by the owner and shall be crushed shell, wetted and compaction rolled. |
| 2100 | Pilings Pilings shall be round tapered wood creosote. See structural drawings for pile specifications and locations. |
| 2150 | Layout Builder shall be responsible for piling layout locations in accordance with the structural drawings. Corner pilings shall be checked diagonally for squaring before setting all pilings. |
| 2170 | Footings Footings shall be dug for all stair posts (8x8) and for HVAC platform posts. See structural drawings for details. |
| 2220 | Waterproofing Front porch and deck above screen porch to be waterproofed with Sealoflex in accordance with manufacturer’s specifications. |
| 2300 | Termite |
| 3146 | Deck Entry Stairs and Decking – Geodeck from Kadant Composites, Inc.(sold at Builders First on Edisto Island or can be found in Beaufort or Charleston). Rail—Timbertech from rayco Building Supply in Beaufort. Decking fasteners-Tigerclaw TC-3 hidden fasteners from Home Depot. |
| 3147 | Deck Labor |
| 3151 | Frame 1st Floor |
| 3152 | Frame 2nd Floor |
| 3153 | Roof Metal roof: V5 with Kynar 500 color finish. |
| 3466 | Slab Concrete |
| 3620 | Plumbing Water Heater: Tankless electric water heater; minimum 3 gallons per minute at 115 degrees F |
| 3710 | Electrical 12KW push button start propane powered electric generator to run: Freezer Fireplace Blower Refrigerator Grand Room Outlets Kitchen lights & microwave Master bath lighting Exterior stair lighting |
| 3730 | Security |
| 3810 | HVAC Heat pumps – dual units 1st floor 2nd floor & attic floors w/electric strip heat added in fan unit Mount compressor units on elevated stilt platform. |
| 4010 | Roofing Materials |
| 4120 | # Fireplace Materials Grand Room – Majestic Ventless with blower—requires buried propane tank (outside door to garage). Tank to be sized for emergency generator as well. |
| 4130 | Brick, Sand and Mortar |
| 4134 | Brick Facing Labor |
| 4510 | Windows |
| 4540 | Exterior Doors |
| 4550 | Int. Trim and Doors |
| 4560 | Sliders, spec. windows |
| 4570 | Garage Door, Opener |
| 4590 | Install Wind and Ext Doors Windows-Remove the divided lights (grids) from all windows. Spec the windows as Weathershield or equal (hurricane glass with exceptions noted below). No hurricane glass on front door and rear porch (down) and deck (up) windows and doors— these opening to have manually operated shutters. |
| 4720 | Ceiling Insulation |
| 4730 | Wall Insulation |
| 4800 | Siding, Cornice, Shutters Siding to be James Hardie ColorPlus Hardiplank-lap siding Select CedarMill, 5” exposure. All exposed trim to be composite such as Harditrim or Primetrim (GP). Material to be purchased with prime & to receive one coat of paint (Benjamin Moore) before installation. |
| 4816 | Siding and Shutt |
| 4817 | Cornice |
| 4900 | Exterior Paint |
| 6000 | Drywall |
| 6130 | Vinyl and Carpets Interior stairs- Pine treads painted with a carpet runner |
| 6150 | Wood flooring Brazilian Cherry on 1st floor, satin finish Red oak #2 on second floor, satin finish |
| 6210 | Interior Trim |
| 6235 | Stairs |
| 6240 | Int. Hardware Satin Nickel Finish: Front door – Baldwin Logan Handset 5335.150 Entrance $ 190.00 Passage – Baldwin 95455.150 Passage 54.00 Privacy – Baldwin 95455.150 Privacy 60.00 Door Bell – Baldwin small oval - 23.00 All butts (hinges) satin nickel finish |
| 6250 | Interior Trim Crown 1x6 on the wall and 1x4 on the ceiling - MDF is acceptable for all trim Baseboard: downstairs 10” MDF with electrical outlets in the baseboard Upstairs 4” MDF Windows: 1x6 head 1x4 bottom with sill 1x4 side (see master bath details on plan) Doors: 1x6 head 1x4 side |
| 6300 | Ceramic Tile |
| 6400 | Cabinets Kitchen – Kraftmaid or equal – ball bearing slides Mapleton Square – no pulls All plywood construction Bisque glaze Bathrooms Master-2 drawer bank far right of sinks similar style to kitchen drawers (simple) Upstairs-Cabinets similar to kitchen (simple) Closet – plastic laminate cabinets with drawers |
| 6500 | Appliances Disposal – Insinkerator ¾ HP $ 132.00 Dishwasher – Whirlpool GU2500 XTPS 597.00 Compactor – Whirlpool 565.00 Range – JennAir cooktop/oven compo. 1,750.00 Microwave – by owner Refrigerator – Whirlpool KTRC22KMSS 1,058.00 Freezer – Whirlpool LFFH2067DW 487.00 Washer – Whirlpool LSQ9010PW 367.00 Dryer – Whirlpool LER8620PW 347.00 |
| 6600 | Plumbing Trim Steam in MBR Shower: Mr. Steam MS225T or equal Stainless tank inside & out 7.5 KW 240V Shower 7’ ceiling Post cooling auto flush & drain Brass head outlet Tempo Plus Controller-chrome (allow $1,700. material) |
| 6700 | Countertops Solid surface Corian or equal in kitchen and master bath (top and skirt). Cultured marble and molded sink in upstairs bath. |
| 6710 | Light Fixtures and Fans Great Room: Hunter 52” Original White 23856 $ 160.00 Others: Hunter 36” Original White 88.00 Porch: Hunter 36” Original White for outdoors 100.00 Exterior Lighting on timers with motion detector switches |
| 6730 | Security Electric Trim |
| 6800 | HVAC Trim |
| 6900 | Interior Paint |
| 6930 | Wallcover |
| 7100 | Landscaping |
| 7180 | Lawn Irrigations |
| 7200 | Pour Walks, Patios, Drive |
| 7340 | Bath Accessories Steam Shower Door 2066; -Alumax or equal 4 inch curb Towel bars, soap dishes and tp holders to be brushed nickel selected by owner. |
| 7341 | Splashbocks, Screens |
| 7400 | PunchOut |
| 9020 | Inspections |

Backsplash in kitchen:

Tile

Miscellaneous:

Master Bedroom-Remove screen porch and extend the room.  Change the windows (in the

extended area) to (2) 30x40-eliminate the door.

|  |  | Finish Schedule |  |  |
| --- | --- | --- | --- | --- |
| 1st Floor Area | Flooring | Wainscoat | Walls | Ceiling |
| Front Screen Porch | Mexican tile* | n/a | hardiplank | plywood painted^ |
| Master Bedroom | Brazilian Cherry Hardwood | n/a | painted^ | painted^/crown molding+ |
| Master Bath | Brazilian Cherry Hardwood | 54" tile | painted^ | painted^/crown molding+ |
| Steam Shower | Tile^^ | n/a | tiled | tiled |
| Master Closet | Brazilian Cherry Hardwood | n/a | painted^ | painted^ |
| Rear Screen Porch | Mexican tile* | n/a | hardiplank | plywood painted^ |
| Utility Room | vinyl sheet | n/a | painted^ | painted^ |
| Kitchen | Tile** | tile for backsplash | painted^ | painted^ |
| Dining Room | Brazilian Cherry Hardwood | n/a | painted^ | painted^/crown molding+ |
| Grand Room | Brazilian Cherry Hardwood | n/a | painted^ | painted^/crown molding+ |
| Powder Room | Tile ** | n/a | painted^ | painted^/crown molding+ |
| Stairs Up | painted | n/a | painted^ | painted^ |
| Basement Stairs | painted | n/a | painted^ | painted^ |
| 2nd Floor Area | Flooring | Wainscoat | Walls | Ceiling |
| Loft | #1 & #2 Oak Hardwood | n/a | painted^ | painted^ |
| Hallway | #1 & #2 Oak Hardwood | n/a | painted^ | painted^ |
| Bedroom 2 | #1 & #2 Oak Hardwood | n/a | painted^ | painted^ |
| Bath | basic 4"x4" tile | n/a | painted^ | painted^ |
| Bedroom 3 | #1 & #2 Oak Hardwood | n/a | painted^ | painted^ |
| Deck | Wood Composite | n/a | painted^ | painted^ |
| Mexican Tile* | Saltillo/Sealed Natural w/light grout @ $2.35 SF |  |  |  |
| Tile** | ceramic/yet to be determined |  |  |  |
| Painted surfaces ^ | Sherwin-Williams |  |  |  |
| crown molding+ | 1x4 ceiling/1x6 wall |  |  |  |